Analysis of lidar-derived snow depth over Fortress Basin

Goal: Determine the accuracy of lidar-derived snow depth (in comparison to manually collected snow depth)

Methods:

- Processed lidar point clouds into 1 m resolution tifs (Figure 1)

- Subtracted the bare ground survey (October 19, 2022) from all snowy surveys to retrieve snow depth rasters

- Used GNSS point locations of manual snow depth measurements to extract lidar-derived snow depth for comparison

Results:

- Fortress Ridge lidar snow depths are in agreement with manually measured snow depth (bias of 0.00m, Table 1)

- Fortress Ridge South lidar snow depths have the largest disagreement with manual measurements (lidar depths are 14.6 cm lower than manual measurements)

  - This is displayed clearly in Figure 2, where FRS is the site with the largest discrepancy between manual and lidar snow depths

  - Largest discrepancies between manual and lidar snow depths appear within the first half of the transect (Figure 3)

- For all survey points across Fortress, for all days, lidar snow depth is biased 6.8 cm shallower than manual measurements

Table 1. Comparison of manual snow depth measurements and lidar-derived snow depths.

| Site | Lidar Snow Depth Bias (m) | Lidar Snow Depth R2 |
| --- | --- | --- |
| Bonsai | -0.055 | 0.946 |
| Canadian Ridge | -0.071 | 0.934 |
| Fortress Ridge | 0.000 | 0.666 |
| Fortress Ridge South | -0.146 | 0.682 |
| Basin-wide | -0.068 | 0.939 |

Lidar snow depth (m)

Figure 1. Example of lidar-derived snow depth (retrieved 20 April 2023). Location of meteorological stations displayed as a black dot.

Figure 2. Manually measured snow depth vs lidar-derived snow depth, coloured by date of retrieval.

In situ – lidar snow depth (m)

Figure 3. GPS locations of manual snow depth measurements at Fortress Ridge South, coloured by discrepancy between manually measured and lidar-derived snow depths (blue = manual > lidar, red = manual < lidar).

Appendix

Dates used for analysis